Hardware Setup & Project Instructions

Dear Students,

Based on the provided wiring diagram, I have prepared a prototype board with all components connected to streamline your initial work. To properly interface with the breadboard, please follow these wiring instructions for the first left column/row of the breadboard:

- VCC (3.3V): Connect to Raspberry Pi Physical Pin 1.

- GND: Connect to Raspberry Pi Physical Pin 9.

- SDA (I2C): Connect to BCM Pin 2 (Physical Pin 3).

- SCL (I2C): Connect to BCM Pin 3 (Physical Pin 5).

- DHT11 Signal: Connect to BCM Pin 21 (Physical Pin 40).

System Access

The Raspberry Pi is ready for SSH access via the local network:

- IP Address: 2.5.5.1

- Login: student

- Password: 1234

- File Location: Basic drivers for the DHT11 and ADS1110 can be found in /home/student/Codes.

Critical Safety Warning: LM35 Temperature Sensor

I have attached the datasheets for the LM35, DHT11, and ADS1110.

IMPORTANT: The pinout diagrams for the LM35 in some documentation can be misleading! Looking at the flat side of the sensor from left to right, the pins are: 1. VCC (Power) 2. Signal (Output) 3. GND (Ground)

+2

Warning: If you swap the VCC and GND pins, the LM35 sensor will burn out immediately. Please double-check your connections before powering the system.